WE930

GPS VEHICLE TRACKER

USER MANUAL

V2.0

Product overview

·Real-time tracking

·Driving behavior analysis

·Over-speed alert

·Removal alert

·Power-supply-cut alert

·Low power alert

·Vibration alert

·Geo-fence

·SOS

·pickup

Standard Parts List

| Item |  |  |  |  | Quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WE930 |  |  |  |  | 1 |  |  |
| Power cable |  |  |  |  | 1 |  |  |
| Relay |  |  |  |  | Optional |  |  |

Specification

| Network |  |  |  |  | 4G/LTE (Cat.1) & 2G |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indication |  |  |  |  | Power (Red) GSM (Green) GPS (Blue) |  |  |
| Built-in battery |  |  |  |  | 300mAh |  |  |
| Measurements |  |  |  |  | 80*32*15.1mm |  |  |
| SIM Card |  |  |  |  | Micro SIM |  |  |
| Operating temperature |  |  |  |  | -20°C to +70°C |  |  |

Product Setup

- Prepare a Micro SIM card that supports the same network with this device.

- Slide the device cover up to open it.

LED Indication

All indicators work for 3 minutes after power on and automatically enter sleep mode when static. Vibration will wake up the indicators again.

Red LED -Power status

| Solid bright | Normal working (External power supplying) |
| --- | --- |
| Off | Power off |

Green LED -GSM status

| Slow flashing | Searching Network |
| --- | --- |
| Solid bright | Normal working with GSM network |
| Off | No SIM card inserted or dormant |

Blue LED -GPS status

| Slow flashing | Searching GPS signal |
| --- | --- |
| Solid bright | Normal working with GPS fixed |
| Off | Dormant |

Interfaces

4 Wire Standard Version

| Interface | Color | Description |
| --- | --- | --- |
| B+ | Red | Power + (9-90V) |
| GND | Black | Power - |
| ACC | Orange | Vehicle ignition detection |
| Relay | Yellow | Vehicle fuel supply cut-off |

Wiring Diagram

Tips for finding right wires:

- Use a multimeter to identify the positive and negative terminals of the vehicle battery.

Note: The current battery voltage will be displayed on the multimeter regardless of whether the ignition key is in the ON or OFF position.

- To locate the ACC wire: Connect the black probe of the multimeter to the negative terminal, and the red probe to a random wire. If the multimeter displays 0V, turn the ignition key to ON. If the supply voltage appears, you have identified the correct ACC wire.

- Connect the two connectors together. If the vehicle does not have a connector, connect the device wires to the corresponding vehicle wires.

Installation recommendation

Please install the device under the guidance of professional personnel.

Note

- The device should side up to sky.

- Metal thermal barrier or heating layer, which are always installed on windshield, may affect the signal, please avoid installing the device under these objects.

Troubleshooting

| Type | Use |  |
| --- | --- | --- |
| Unable to connect to platform | Check APN and IP settings Check whether the data of SIM card is enabled Check SIM card balance |  |
| Tracker shows offline on platform | Check external power is still connected Check if the vehicle entered network blind area Check the balance of SIM card |  |
| Unable to locate | Make sure the top side facing upward without metallic things shielded Make sure area with satellite coverage |  |
| Location drift | Area with poor GNSS signal, drifting may happen Check whether vibration happens around to trigger the accelerator |  |
| No command reply | Make sure command format is correct. Device must be in network coverage area Make sure SIM card inserted well and with SMS service |  |